VALVE GEAR PROGRAM INSTRUCTIONS RUNNING IN MICROSOFT WINDOWS

These Windows valve gear programs were written betweem 1998 and 2005 by

Charles J. Dockstader HCR 69 Box 3111, California Valley, CA 93453-3111

805 475 2519  cjdvalvegear@tcsn.net    www.tcsn.net/charlied

The programs have been placed in Public Domain and are FREEWARE. The

programs can be copied and distributed.

The author accepts no responsibility for any claim arising from the use of these programs.

The following instructions are not complete or final.

NOTE ***********

A problem may exist where the slide bars are masking the displayed data.

Windows has a setup available that is called Large Fonts and it changes the scaling of fonts causing the problem. It is at RightClick DeskTop/Properties/Settings/Advanced/ (font size). This can be set to Small Fonts (the default) and the computer restarted.

Or in the program Setup an item called SetLargeFonts will change the scaling of the text position so the displayed data is not under the slide bars.

NOTE ***********

Each valve gear has an INSTRUCTIONS SPECIAL file that may contain special instructions for that valve gear.

SCROLL BARS

A normal scroll bar has a small change when an end button is clicked, a larger change when an area between the end button and the slide is clicked and a greater change when the slide is dragged with the mouse. Using a normal scroll bar, the slide will stay where it is placed when the mouse button is released. The normal scroll bar has fixed limits and can move a variable between these limits.

The scroll bars used for the pan, zoom, reference points and dynamic variables operate in a slightly different manner. All these functions need fine adjustment with some needing large value changes. The scroll bars have a fixed limit of change but the slide will return to the center when released. Thus the value can be changed beyond the normal limit of the scroll bar by moving the slide several times, releasing it between each move. If the slide is moved slowly, the final value will have a larger change. If the slide is moved quickly, the value will have a smaller change.

To set the engine in midgear reverse, first click CENT button, then click the left slde bar button.

REFERENCE POINTS

Reference points are used to mark or to measure dimensions on the graphics motion screen.

RefPoints brings up the 2 referance points and four scroll bars that will move the reference points horizontally and vertically. Also the two buttons RP1 and RP2 are turned on. These buttons pan the graphics so a referance point is moved to the center of the screen. Thus making it easier to zoom in on a referance point. The left mouse button when clicked on the graphics motion screen will move the reference #1 to that mouse cursor position on the screen. RefPoints are not active above the lower edge of the reference data. The right mouse button will move the reference #2 in the same manner.

PAN AND ZOOM

The 'Norm' button will pan and zoom the graphics to the initial starting point of the internal dimentional data called Nominal. Buttons 1 to 6 are preset buttons and have an initial preset. If pan and zoom are used to position the graphics motion and this position is to be used again, first click the 'P' button and then click one of the six preset buttons, which sets that preset button to the present view.

To save preset settings--

After setting the preset buttons, click the 'Norm' button. Click the 'Save As' button. Select the destination folder. Enter the name for the file to be saved. Click the 'Save' button.

After restarting the program, click the 'File Open' button. Select the folder containing your saved file. Select the file. Click 'Open'. Click the preset numbers from 1 to 6.

If the scale has been changed or the main desired view is off screen, then set up preset '1' for your main view. Save dimentional data file with this view present. When restarting the program, open the desired dimentional data file and don't use the 'Norm' button. Also if scale or units have been changed, check the two scaling factors, correct if necessary and resave dimentional data file. See SCALING FACTORS.

To display several Valve Gears--

Start the first Valve Gear. In 'SetUp' select 'SetScreenDataOff'. In 'SetUp' select 'SetPanZoomOn'. Pan and zoom the image to the upper left corner. Drag the lower right corner to set the window size. Drag the window to the right and position it. In 'SetUp' select 'SetPanZoomOff'. Start the second Valve Gear. Repeat the pan and zoom procedure used for the first Valve Gear. Reopen the first Valve Gear.

CHANGING DIMENSIONS

The dimensions are in the Edit/FrameDim or CylDim boxes. The dimensions are changed using the Windows procedure. A single click on the value will allow the value to be modified using the BACKSPACE or DELETE keys, or a digit will be added or overwritten depending on the INSERT key. A double click will change the background color and the new value must be entered. The changed value is NOT ACTIVE until the next variable to be changed is clicked or the Enter key is clicked or the Close button is clicked.

Changing a dimension can activate the Error Beep and the visual ERR icon if the dimension is too large or too small causing the calculation to make an error. If the scale is being changed, the Error Beep will sound and the visual ERR icon will display until all or most all dimensions have been changed to the new scale. The Error Beep and visual ERR icon can be turned off at the...  Main Menu / Setup / SetErrorBeepOff.

In the FrameDim and CylDim boxes there is a small button to the left of each item. Clicking this button will bring up a graphic display showing the position of the selected dimension. Clicking again will return to the main graphics.

DYNAMIC VARIABLES

To activate a Dynamic Variable, the value of the Variable must be clicked in Edit/FrameDim or CylDim to display the variables. Bring up the display of up to 4 Dynamic Variables from the Main Menu or Tool Bar.

The adjacent Scroll Bars can be used to change the values of the Dynamic Variables. The Scroll Bars will increase the value when the bar is dragged to the right of center with the mouse and decrease when dragged to the left of center. When the mouse is released, the Scroll Bar will return to its center position. Picking the End button of the Scroll Bar will change the variable by 0.001 units, 0.1 units for angles. Holding the mouse button down will step the variable in 0.001 units, 0.1 units for angels. Picking the area between the end button and the bar, the change will be 0.01 units, 1.0 units for angles. Holding the mouse button down will step the variable in 0.01 units, 1.0 units for angles.

Note:  Variables ReverseArmNorm and ReverseArmMax are not dynamic.

UNDO VARIABLE CHANGES

The menu UnDo or the tool bar UnDo will turn on / off the UnDo button and its display of the stack number, name of the variable, present variable value and old variable value to be restored. The UnDo stack will store the last 99 variable changes. When the UnDo button is clicked, the last changed variable is restored to its previous value. Each secussive click will restore the next variable in the stack. If more than 99 variable changes have been made, the UnDo button will restore only the last 99 changes.

DRIVER ANGLE

The driver angle can be set to any angle using the menu Setup / SetDriverAngle Box, or the Driver Angle Tool on the Tool Bar. When the driver is stopped the DriverAngle scroll bar moves the driver angle from 40 degrees below 0 degrees to 40 degrees above 360 degrees. The end buttons move the driver in small increments equal to the setting of the Step Angle Scroll Bar.

STEP ANGLE

The step angle can be set to any increment from 0.01 degrees to 45.0 degrees using the menu Setup / SetStepAngle Box or the StepAngle Tool on the tool bar. The StepAngle scroll bar will set the step angle to  predetermined values between 0.01 degrees and 45.0 degrees.

STEAM DISPLAY AND EXHAUST SOUND

If the step angle is greater than 10 degrees, the exhaust sound beat could be very uneven or absent due to the large step. The step not coinciding with the opening of the exhaust port. The sound beat is produced at the first step after the exhaust port opens. Also since Windows takes controll of the system, mouse movement has an effect on the exhaust sound beat.

PRINTING INSTRUCTIONS

To print the instructions start at the desktop. Select Start / Programs / Accessories / WordPad / File / Open.  Navigate to the folder containing the Valve Gear programs. Select  Instruct.doc and open. Select File / Print.

PRINTING A SCREEN

To print a screen first select the desired screen. If the screen is the motion screen, select Stop and set its position as desired, driver angle, reverse arm position, pan and zoom, screen data off, etc. Select Alt - PrintScrn. This places the screen into the ClipBoard. From the desktop select Start / Programs / Accessories / Paint / File / New / Edit / Paste / Yes / File / Print / Ok.

SCALING FACTORS

There two scaling factors in the programs. They may require adjustment before saving the dimensional data or changing model scale or using metric dimensions or using the programs.

The first involves the vertical scale of the Sine and Oval diagrams and can be adjusted while starting the Sine or Oval diagram.

The second involves the graphics display of the dimensions in the dimensional menus using the small button. If scale or the unit of measurement has been changed, the graphics display of the dimension moves off screen. Go to EditDimensions / SetGraphicZoom  to adjust the zoom factor. Then check the graphics display of several dimensions.

USING DRAW LINES AND CIRCLES

The intent of this function is to draw points of interference on the valve gear motion screen so one sees this interference while laying out the valve gear. Only a few types of lines can be drawn.

The function uses the Reference Points as its basic input. To draw a line on the screen click ReferencePoints. Position the cursor on the screen for the begining of the line. Use the left mouse button to click the begining of the line. Position the cursor for the end of the line. Use the right mouse button to click the end of the line. Click DrawLinesCircles. The DrawLinesCircles dialog box will appear. Click Type Line. Click desired color. Click Referenced to 0,0. Click DrawClose. The graphics motion is then returned, showing the line.

The reference points can also be set using the reference point slide bars or the SetUp/SetRefPoints dialog box.

This line is placed on the screen with both ends referenced to the 0,0 point (main driver center). The line does not move with any pivots.

Using Type Line, Referenced to Pivot1  will draw a line with both ends of the line referenced to the selected pivot. When a Pivot is the Referenced condition, the program will return to the graphic motion screen so a pivot can be selected. Use the left mouse button to select the pivot. The motion should be stopped and adjusted so the desired pivot is clear of other pivots. After the pivot has been selected the progam will return to the DrawLinesCircles dialog box for final action. Both ends of the line will move in a horizontal/vertical motion following the selected pivot point, not in an angular motion.

Using Type Line, Referenced to Piv1/0,0  will draw a line with the beginning of the line referenced to the selected pivot point using left mouse button and the end of the line referenced to the 0,0 point using the right mouse button.

Using Type Line, Referenced to Piv1/Piv2  will draw a line with the beginning of the line referenced to the first selected pivot point using the left mouse button and the end of the line referenced to the second selected pivot point using the right mouse button.

Using Type Circle, Referenced to 0,0  will draw a circle with its center at the reference point using the left mouse button. The center of the circle is referenced to the 0,0 point. The circumference of the circle will pass thru the point using the right mouse button. The radius of the circle is the distance between the two reference points.

Using Type Circle, Referenced to Pivot1  will draw a circle with its center at the reference point using the left mouse button. The center of the circle is referenced to the selected pivot. The radius of the circle is the distance between the two reference points.

NOTE TypeCircle and Referenced to Piv1/0.0 or Piv1/Piv2 are not valid circles.

Dimensions can be changed in the table of dimensions. These dimensions are referenced to the selected reference point for that line or circle. Use Close to return to the graphic motion.

To place lines and circles at exact dimensions, it is easier to place them in their approximate position using the mouse pointer, Then reenter the DrawLinesCircles dialog box and change the dimensions to the final value in the table of dimensions.

CAUTION The HighLighted Line must be on a blank line for entering new information or on the desired line for clearing a line. To change click the desired item number at the left of the line.

To clear an item, click on the item number, then click ClearLine. Use Close to return to the graphic motion.

There is a demo file that can be loaded in the A. J. Stevens 1885 valve gear showing some drivers, frame, crosshead and their relation to the valve gear.

USING PIVOT DATA

Pivot Data will display the XY coordinates of up to 6 pivot points on the screen. On some of the programs some other points are also included but are not marked. Selecting PivotData from the main menu or the icon on the tool bar will turn on the Reference Points and the Pivot Data. Using the L Mouse pick one of the six line numbers. The Vert Horz will turn on displaying 0.000. Next pick a pivot. The program will select the closest pivot. If pivots are too close, stop the motion and zoom in on the desired pivot. Picking the same number will overwrite the first pick on that number. The selected pivot will be labled with the line number. The XY coordinates are displayed in the Vert Horz columns and are the dimensions from the main driver 0,0. If two adjacentline numbers are used, a 5th dimension will appear to the right of the XY dimentions and is the dimention between the two selected pivots.

The PivotData button below the screen data will turn on a table which will display all selected pivot data for one valve gear cycle. The cycle starts at 0 degrees and ends at 360 degrees using the step angle as its increment. A step angles of less than 1 degree will not be a complete cycle. The mouse can be used to select portions of the table or using the menu Edit SelectAll will select the complete table. The Edit Copy will then copy the table to the ClipBoard. Or R Mouse will also bring up SelectAll and Copy.