KLOSE SPECIAL INSTRUCTIONS

Klose built articulated narrow gauge locomotives in late 1800 and early 1900. These locomotives used swivel axles and a lozenge linkage to adjust the effective side rod lengeths as the swivel axles moved to negotiate sharp curves.

Only a few of the actual dimensions are known. The dimensions are in CM and were scaled from a small drawing. The actual system would have two closed loops that would bind the links and levers and would be adjusted so the binding would be less than the bearing clearances. The simulation leaves these two loops open so the calculations do not bind.

Some assumptions were made with the valve gear and cylinder. No cylinder was shown. The valve is on the side of the cylinder. The expansion link is short and straight. The three pivots for the lifting link and eccentric rods are on the outer side of the expansion link. The link block, connected to the valve stem, is on the inner side of the expansion link. The link block slides in a vertical slot of the expansion link as the expansion link is moved vertically.

The locomotive being narrow gauge, articulated and for sharp curves would not be for high speed. The valve gear will not give good results at full gear and mid gear. The valve gear would then be adjusted for best results at full gear forward and reverse, not mid gear.

This program does not compute the mid gear position of the reverse arm and is adjusted by the length of the lifting link..