SINGLE ACTING ROTARY VALVE SPECIAL INSTRUCTIONS

One design of this gear uses a wide Valve Steam End Ang and Valve Exhaust End Ang that are opposite each other. The valve being a cylinder with two flats machined on the opposite sides. This makes a very simple valve design. The Cylinder Port is then two ports with a bridge between them.

To do a design, first set up the desired basic frame and cylinder dimensions and save them in a file. It is a good idea to save a series of files. Adjust the Eccentric Rod so the Valve Lever has about equal throw to either side.

In the Sine Dia adjust the Fwd and Rev Ecc so they cross at the Zero Degree Line. Adjust the ValveLever-SteamEdge Ang so the Steam Port Open line passes thru or below the crossing depending lead desired. Lead is listed at the right side of the Sine Dia.

In the Oval Dia note the cutoff point as the oval crosses the Steam Port Open line. If this is too high go back to the Sine Dia and move both ecc so the open peaks are closer together. Check the Oval Dia for cutoff.

To adjust for normal Lap, in the Sine Dia adjust the Valve Steam End Ang so the Exhaust Port Open line has 180 degrees between the two crossing points of the Fwd sine plot.

Try different lengths of the Eccentric Rod. This will change the slope of the sine plot.

Try different lengths of the Valve Lever. This will change the amplitude of the sine plot.