ALLAN OUTSIDE ADMISSION SPECIAL INSTRUCTIONS

The ALLAN gear in this program uses direct motion for the radius rod from the link to the valve stem (no rocker). The exp link, radius rod and the valve stem are on the main driver-cylinder center line. The valve is on the side of the cylinder.

Allan gear is also used with the valve on top of the cylinder.

This form had the link, radius rod and valve at an enclined angle with its center line passing through the the main driver. The valve seat being parallel to and below the enclined angle center line.

To use this form in the program, one must use some Trig to convert the enclined angle to a horizontal zero angle. Several points must be rotated about the main driver. Rotate the two eccentric angles by the enclined angle. Rotate the valve and valve seats so the effective distance from the main driver to the center line of the exhaust port is equal to the distance to the cylinder center. The reverse and lifting arms pivot weight shaft must be rotated about the main driver. The connecting point of the reach rod to the reverse arm would also have some effect. This is the point that the program uses for Reverse, Center and Forward. The final positions of these points would be adjusted by the notches on the quadrent for the reverse

lever in the cab.

If the weight shaft is placed above the exp link then the two lifting links are made negative. The vertical dimension for the weight shaft pivot becomes positve.

If the eccentric rods are crossed, then the two eccentrics are rotated for the proper angle. This exchanges the reverse-forward

positions on the reverse arm. Other characteristics also change.

If the weight shaft is behind the exp link  then the two lifting arms and the radius rod short end are all negative.