GRESLEY INSIDE ADMISSION SPECIAL INSTRUCTIONS

Gresley is a three cylinder engine using a Walschaert valve gear for both

sides. The left side crank lags the right side crank by 120 degrees. The

center cylinder is at a slant with its crank lagging the right crank by

240 degrees plus the slant angle. The valves are in a horizontal plane

with the center valve on the left side of its cylinder. The center valve

is operated using conjugated levers connected to extensions from the right

and left valves.

This program uses many common dimensions between the cylinders and valve

gears, not allowing individual adjustment.

To obtain six exhaust beats for each revolution of the drivers, set the

step angle to 3 degrees or less. The exhaust beat occures when the exhaust

port is opened at the angle produced by the step angle. This timing can

be late in degrees by the step angle plus any time added by Windows doing

its maintenance.